Software diskette #6

======================

1.Programmer Files :

EPLD1.EXE  : ALTERA EPLD 310,320,600,900

TI 610,910

SEEP1.EXE  : NS 9306/07/46,93CS06/26/46/56/66.

XC1736/A,MB8541P.(SEE 2. REVISION)

BPPGM.EXE  : BIPOLAR PROM NON-register type

32x8 to 4096x8,

CYPRESS CY7C291/292,27C291/292...

EEP1.EXE   : GENERAL EEPROM programmer software.

2816,2816A,2817,2864A,28256A,28F256,28F512

2.REVISION of programmer software :

Hardware Revision on  :

VERSION 1 : 1ST VERSION

VERSION 2 : 2ND VERSION 1989/11/5

TO SOLVE Z8 MPU SERIES PROBLEM,

1.UPPER BOARD Q9 DELETED,

2.UPPER BOARD R18 CHANGED TO 47 OHM 1/2 W.

3.UPPER BOARD R168 CHANGED TO 47 OHM 1/2 W.

CHANGE ABOVE COMPONENTS WIIL NOT DISTURB

OTHER DEVICES PROGRAMMING.

Hardware Revision on  :

VERSION 1 : 1st version hardware.4/15/1990.

current limit resistor from 22 ohm

to 10 ohm.

VERSION 2 : 2nd version hardware.12/1/1990.

Add GND control for PIN 30 to solve

40 PIN word-wide EPROM problom.

EPLD1.EXE V3.00 : 1st released.

V3.14 : 1/1/91.

V3.16 : 4/1/91.

V3.17 : 5/1/91, Revision of Blank check.

V3.18 : 7/1/91, Revision of text on the screen.

Revision of NON-TURBO ADDER.

Add INTEL 85C224,5AC312.

V3.19 : 7/3/91, Revision of text on the screen.

V3.20 : 7/11/91,Revision of device type for ALTERA Mfr.

Improve INTEL 85C312 programing ALGO.

Improve TI,ALTERA EP330/630 programing pulse.

V3.21 : 8/6/91, Revision of text on the screen.

Revision of programming voltage for TI EP330/630.

V3.23 : 12/10/91,Add ID code for EP330.

V3.24 : 12/19/91,Revision of programming voltage for TI EP330/630.

V3.25 : 1/15/92, Add ALTERA EP910-30T/-35T/-40T.

Revision of programming voltage for TI EP910.

SEEP1.EXE : V3.00 : 1st released version.

V3.03 : for EXPRO-40 hardware modification as follows :

1.During NS93 & XC1736/A series PROM programming,

user has to connect pin 11 to pin 21,

and connect pin 28 to pin 24.

2.During 8541 series PROM programming,

user has to connect and connect pin 28 to pin 24.

V3.08 : 1/1/91.

V3.10 : 2/8/91, Add OKI MSM16911RS

Add MATSUSHITA MN12C201D(must short TEXTOOLS

PIN14 and PIN21).

V3.11 : 3/27/91,Add XICOR X24C04,X24C16.

V3.12 : 4/1/91, Add ICT 93C46/A,93CX46,93C56/CX56,93C66/CX66.

V3.14 : 7/1/91, Add SAMSUNG 93C06,93C07,93C46. XILINX XC1765.

XICOR X2402/I,X2404/I,X24C01/I,X24C02/I.

V3.15 : 8/17/91,Revision of programming algorithm for

XICOR X24C01/I.

V3.16 : 10/1/91,Add XICOR X2444,X2444I,X24C44,X24C44I,X24C45,

X24C45I.

SIEMENS SDA2506,SDA2516.

SGS-THOMSON ST24C02A,ST25C02A,ST24C04,ST25C04

ST93C06,ST93C46A.

V3.18 : 11/15/91,Revision of text on the screen.

V3.19 : 1/15/92, Add NS:95C12, AKM:AK93C46,  GI:PCD8572,

PHILIPS:PCF8582E,PCF8582A, ISSI:IS93C56,

SGS-THOMSON:ST93CS46,ST93CS47,ST93CS56,ST93CS57.

BPPGM.EXE : V1.00 : 1st released version.

V1.01 : Add a powerful screen editor to replace DEBUG.COM,

please ref. section 5.

V1.02 : TI BPROM 18S030/18SA030(32*8)  are added

V1.03 : Improve TI BPROM 18S030/18SA030.

V3.00 : new version.

V3.03 : 1/1/91, Add WSI 191/B,291/B,43/B,49/B,51

V3.04 : 2/8/91, Add TI TMS27C291/292

Improve TI TBP28L22 programming ALGO.

Improve SIGNETICS 82HS195 programming ALGO.

V3.05 : 3/1/91, Revision of programming voltage for WSI WS57C51.

V3.06 : 4/1/91, Update load file.

V3.07 : 5/1/91, Add ICT 27CX321/322,27CX641/642.

V3.08 : 7/1/91, Add MICROCHIP 27HC641, SIGNETICS 27HC641/642,

ASAHI KASEI AK27CX161/162/321/322/641/642.

V3.11 : 8/1/91, Revision of Blank check.

V3.12 : 10/1/91,Add SGS-THOMSON M27HC641,WSI WS57C45,

CYPRESS CY7C245A.

V3.14 : 1/15/92.

EEP1.EXE :  V3.00 : 1st version.1/1/91.

V3.01 : 2/8/91, Add HITACHI HN58C256

Add INTEL   28F010,28F202

Add SEEQ    48F512,48F010

Add HUGHES  H3300

V3.02 : 3/1/91, Revision of text on the screen.

Revision delay time for SEEQ 2816,2817,2864.

V3.03 : 4/1/91, Update load file.

Add ATMEL 28C04,28MC010,28C010,28C1024,

28C191/L,28C292/L.

EXEL XL46C15.

V3.05 : 7/1/91, Update load file.

Add AMD 28F256,28F010,28F020. SAMSUNG 28C256.

SEEQ 52B13/H,52B33/H,2804A,36C16,36C32,

38C16,38C32,47F512,47F010.

XICOR 2804A/I,2816C/I,28C64/I,28C512/I,

28C010/I.

V3.06 : 8/12/91,Add Data Protection function.

XICOR 28C64/C64I,28C512/C512I,28C256/C256I,

28C010/C010I.

V3.07 : 10/1/91,Add ATMEL AT29C256,AT29C257,AT29C010.

INTEL 28F001BX-B,28F001BX-T.

XICOR X2201A,X2001/I,X2004/I/M.

V3.09 : 1/15/92,Add XICOR:2816A/16AI,X88C64/C64I.

DALLAS:DS1220AB/AD,DS1225D/E,DS1230Y/AB.

MITSUBISHI:M5M28F010.

3.NOTE : We will keep updating software for new programmable devices.

Special device will be arranged to be 1st priority, if user offer us

the programming information of that device.

4.Powerful screen editor function description :

Some users said that it is unconvenient to specify the address by segment

prefix form during DEBUG MODE .(such as F000:E000 represent address FE000).

User only want to enter the offset address in memory buffer.

According to upper reasons, we develop a simple screen editor that will

meet the requirment of users.

Although the DEBUG.COM has been replaced, but user still can invoke it

by SHELL( DOT ) command in this editor.

Editor command are almost the same as DEBUG.COM, such as

D,E,M,F. It is not necessary to enter the segment address before

the start address,end address. Only the offset address in memory buffer is

needed.

The extra commands are shown below.

E : Same as E command in DEBUG.COM, except that user can modified

data in full screen editing mode.

There are two editing fields, BINary and ASCII, you can edit

any datas on each field.

P : Same as D command,except that the data will be printed to the

printer.

C : The command will calculate the total sum between start and end

address.

S : Will search the ASCII string that are included between start and

end address.

B : Will search the BINARY data that are included between start and

end address.

. : Will execute the excutable file and pass argument to the file.

This command liked the BASIC SHELL command can invoke any file and

pass controll to it. So you can invoke your own editor to do more

powerful job.

Note : The actual segment address of memory buffer shown on

main menu is for referenced only. You can pass this parameter

to the invoked file for special purpose.

? : Will display the editor commands.

------------------------ END -----------------------------